PS1 Latest Direct Reading MASTER 5 Installation and Usage Instructions 20240922

PS Direct Reading is continuously updated by http://diy918.taobao.com . Our store has long been engaged in PS1 Direct Reading installation services, allowing perfect installation for any PS console.

Modifying the device carries risks, so please proceed carefully. All modification consequences are the buyer's responsibility!

Chapter 1: M5 Direct Reading Installation Method

Chapter 2: Methods for detecting unsuccessful installation after installation

Third Image: Introduction to M5 Related Hardware [M5 Direct Read and M4 Direct Read are installed exactly the same, only the cross-region activation method differs].

M5   M4

M5 Direct Reading Installation Method

1. Installation steps: (Beginners must strictly follow it, experienced users may do as they please)

Before installation, please test whether the host can read discs and play CDs and music. If you are replacing the direct-read, please first take photos of the original direct-reader installation and back up the original installation images.

- Prepare the straight-reading chip, wires, double-sided tape, soldering iron, and screwdriver.

- Determine the model and version number of the PS console. (If it's a reassembled or refurbished phone, the model and version may be incorrect; you can refer to the following information to identify it yourself.)

- Disassemble the main unit and take out the motherboard.

- Touch metal railings, guardrails, etc., to remove static electricity from your body. It's best if the soldering iron has a built-in grounding wire to prevent damage to the reading from static electricity (the possibility is low, but not ruled out). Therefore, it's recommended to apply static electricity before soldering as a precaution.

- For direct reading, first solder the wires. [Recommended to use thin or flexible wires]

- Set the boot version jumper. (Simply solder the short-circuit jumper wire; see the table below for corresponding selections)

The Japanese version shorts JPA, the European version shorts EUR, and neither JPA nor EUR is the US version.

- Attach double-sided tape at appropriate positions on the motherboard and secure the vertical reader.

- According to the motherboard point diagram, solder the wires properly. The wire length should be relatively short; too long will affect the guiding effect (try to make wire soldering points smooth and free of burrs).

Point Description 1: Point B is dedicated to the first-generation SCPH-1000 machine; see details later.

Point Explanation 2: In some images, one point has two arrows, and the two arrows refer to the same point. Select any point to weld.

- After installing the motherboard and the bald connector, I powered on and tested it.

Boot version selection:

| Japanese version machine | SCPH-1000、3000、3500、5000、5501、7000、7500 9000, 100 BIOS in Japanese | Japanese version guide |
| --- | --- | --- |
| US version machine | SCPH-1001、3001、3501、5001、5501、7001、75019001, 101 English | US version guide |
| European version machine | SCPH-1002、3002、3502、5002、5502、7000、75029002, 102 English | European version guidance |
| Hong Kong version of the machine | SCPH-1003、3003、3503、5003、5503、5903、7003 7503, 9003, 103 in English | Japanese version guide |

Installation example images: (Taking the SCPH-7500 Japanese version as an example) Note: The installation kit includes a large installation image

1. Solder the wires properly

2. Set the boot version jumper correctly.

3. Find a suitable location and install the direct-reading chip.

4. Solder the straight-reading wire.

5. The weld joints of the wires should be as smooth and burr-free as possible.

2: Installation failure, boot failure, disk load failure, inability to enter the game, etc. Check steps:

Many people find the game unable to run after installation, and many issues are minor issues. I hope everyone carefully and patiently checks every step.

| Cannot read the disc for inspection | You can read the disc to check |
| --- | --- |
| 1. Check whether the direct reading line is short circuited with surrounding circuits, whether the straight read wire is short circuited with the motherboard, and keep the wires as short as possible. | 1. Check whether the boot version jumper is set up, then confirm the motherboard model again. |
| 2. Check whether the bald flex cable is installed properly, whether the barbed flex cable is properly plugged in, and also check if the cable is considered broken. | 2. Test with a genuine game disk. If the genuine version cannot boot and only enters the CD interface, it means the installation was unsuccessful. Carefully check whether the installation location and boot version are correct. |
| 3. Strictly check whether the first soldering point is correct, whether each weld is smooth and burr-free, and check whether each weld point has any short circuits with surrounding solder joints. (This step is important and is a common problem.) | 3. Genuine versions work, but burned discs cannot. If genuine ones can run, it means direct read installation is correct. Pay special attention to checking whether the burned disc is properly burned and if the regions are consistent. Also, many machines that have never installed direct read have worn out the bald head, but can read genuine files, but reading the disc is very difficult. Therefore, you also need to check the aging of the barbed disc. |
| 4. If testing without a top cover, you need to press the door switch on the motherboard to read the disc. | 4. In very rare cases, when installing direct read, certain wiring was disconnected. In such cases, you need to restore the disconnected wires on the motherboard. This is rare, but it does happen—I've encountered it. |
| 5. After clearing the above issues, completely remove direct read, then read the disc to see if it can enter the CD interface. Once it works, reinstall direct read. | 5. If you go beyond these limits, you can contact me for remote analysis of the cause. |
| 6. After re-soldering, if the disk is not read, direct reading may cause welding damage. (The chances are small, but not ruled out) | 6. Still get the red circle; see the RHP issue later. |
| 7. You can contact me to see if there are any other mistakes. |  |

3. Explanations of related hardware, terminology, and materials

Onboard LED indicator lights can show operating status and disc reading status. Depending on the situation, the flicker varies.

- Locked Region: Machines with red text in the table above are locked to the region. To play games in other regions, these machines can only install MASTER Direct Read (M5).

- RHP: When your machine runs the game, the following image appears:

This means your direct reading did not bypass the RHP check. Nowadays, both V7 and M4 can pass RHP, but for 1000-7000 models, due to disc reading issues with the bald head,

Sometimes it might not pass, but you can try several times. Only a very few can get above 7000.

- How to disable direct reading: Before powering on, hold down the RESET key on the main unit. After 1 second, release (do not exceed 2 seconds; see reason 5).

After turning off direct read, the host essentially no longer has direct read, and it runs just like the original genuine machine.

- On/Off Cross-Zone:

Method 1: Short the COD patch cord. COD is the cross-zone unlocking patch cord. After short-circuiting, the device will run cross-zone unlocking every time you boot up.

After unlocking across regions, the PS boot screen will display a screen garblage effect! (Thick phones display SONY) and thin machines lack text, which are normal and only appear on the PS logo interface. After entering the game, everything runs normally.

Method 2: Do not short-circuit COD [default]. Turn on the power switch. Within 1 second after powering on, the direct reading will detect the silo door switch status. If the silo door is closed,

Direct reading operates using the standard guidance method; if the warehouse door is open, direct reading operates using cross-zone guidance. [Open Position Gate - Release - Boot - Close Gate = Cross-region Game Operation].

After enabling the cross-zone function, as long as the power is not turned off, the cross-zone function will remain active. After shutting down, cross-region shutdown is required.

- Special stealth mode: This mode is only available on the first-generation 1000 series machines. Because the servo chips of the first-generation machines are different (chip images are below), ordinary stealth mode still shows red circles if some games fail RHP checks. Therefore, this special stealth mode is needed to ensure all games pass RHP.

Here's how to enable it:

First, when installing direct reading, connect to point B, then power on and run. While running, hold down the reset button. After 2 seconds, release it (you can extend it by 1 second). Releasing the reset button at this point will enable special RHP booting!

4. Host boot version identification:

[Refurbished or assembled machines are mostly models priced above 5500 RMB. 】

If it's an assembled or refurbished machine, you must identify which region the host version belongs to; otherwise, you won't be able to select the corresponding boot version. You can check the servo chip model on the motherboard to confirm the boot version. On the motherboard, there is a 52-pin 4-square patch IC. You can check the first row of numbers to determine the version. The first row consists of C and 4 digits forming CXXXX. The first character of each 4 digits is the version number, and 1 represents the Japanese version boot, such as C 1021。 2 stands for the US version of the guide, such as C 2040. 3 represents the European version of boot, such as the C 3060, so we only need to check this chip to know the corresponding boot version! The Hong Kong version is in the Japan region, so the servo chip numbers on the Hong Kong version also start with 1.

[The last two digits are batch numbers, not related],

Note: The servo chip of the first-generation S CPH-1000 model is 80-pin!

Japanese and Hong Kong versions: C 10XX

US version servo chip: C 20XX

European version servo chip: C 30XX